PROJECT “HETEROTOPIAS”, CO-FINANCED THROUGH INTERREG-IPA PROGRAMME BULGARIA - SERBIA

| OPEN AIR MUSEUM |  |
| --- | --- |
| TECHNICAL SPECIFICATION |  |
|  | CONCRETE TABLE L80 cm, B80 cm, H70 cm, reinforced fiber concrete Surface treatment foundation - custom mosaic, white, UV and waterproofing varnish Surface treatment table top - polished mosaic, white, UV and waterproofing varnish Playing fields 3x1 – chess, Nine men's morris and “Man, Don't Get Angry” |
|  | CONCRETE CHAIR L40 cm, B40 cm, H40 cm, reinforced fiber concrete Surface treatment foundation - custom mosaic, white, UV and waterproofing varnish Surface treatment table top - polished mosaic, white, UV and waterproofing varnish |
|  | CONCRETE BICYCLE STAND L100 cm, B55 cm, H26 cm, high strength reinforced fiber concrete and natural mosaic Surface treatment - custom mosaic, white, UV and waterproofing varnish built-in metal rings from stainless steel |
|  | CHILDREN’S SPRINGBOARD Square springboard, for installation in terrain Standard EN1176, safety zone 23 sq. m, risk of falling – 1 m, Age group – 3+, number of children – 2, opportunity for children with disabilities – yes, frame - hot-dip galvanized steel, Slabs – rubber Euroflex with thickness 30 mm; Trampoline surface – straps reinforced with steel connected to the frame by steel springs – each one of which with L 185 mm and D20 mm |
| SCHEME See list 5/9 of the graphic part of the investment project for Open Air Museum | BENCH PLATFORM L390 cm, B380 cm, H48/40 cm /48cm installation raft on Klotz/ /40cm installation and backrest above the seat / Supporting grate for the wooden surface - carbon or low alloy steel, closed profile 30x30mm, powder coated, fasteners – galvanized, powder coated Indoor installation of concrete walls of Klotz Wooden planks for seat and backrest – D 12cm, H 6cm, lengths - by size of scheme mandatory on-site control measure, Bulgarian spruce, impregnated for 30 years warranty for protection of atmospheric influences, insecticides and fire |
|  | CHILDREN’S TRIPLE SWING Swing – basket, standard swings – 2 pcs. and swings for babies – 2 pcs. Standard EN 1176, safety zone 66 sq. m., risk of falling – 1.55 m, Age group – 2+, number of children – 9, opportunity for children with disabilities – yes, Supporting structure – tubular D125 mm, steel – varnished galvanized, bicolor grey grained coverage with light wavy surface for increased scratch resistance Horizontal beams – unpainted galvanized steel, PVC sliding rings limiting the abrasion of metal parts, with vandal-proof bracket from stainless steel, rotating bearings, chains and fasteners – from stainless steel Seats by age: - a whole piece of rotating polyethylene fixed to a frame from stainless steel, equipped with buffers; - shock-absorbing rubber honeycomb; - polyurethane. |

ARCH. MARIELA ANDREEVSKA